Hardware Design

Lab 6 Report

Peripheral Components:

VGA, Mouse, and Dual FPGA

Team 01

112062122 謝佳晉    112062144 范升維

Table of Contents:

1. Dual FPGA Communication	1

2. Slot Machine	1

3. The Car	1

What have we learned from Lab 6?	1

Contributions	2

1. Dual FPGA Communication

2. Slot Machine

3. The Car

We are asked to modify the sample code to make the car complete the tracks. The sample code are divided into four parts, which are sonic, motor, tracker sensor, and top.

3.1 Sonic Part

In the sonic part, we modified the sonic_top module. In order to let the car stop when the distance between it and obstacle in front of it is less than 40cm, we set stop to (dis < 20’d4000).

The overall design of sonic part is shown in Figure 3.1.

▲ Figure 3.1: sonic part

3.2 Tracker Sensor Part

In the tracker sensor part, we modified the tracker_sensor module. We design our state to be 4bit to include in the STAY_L and STAY_R states, which we use when the sensors are all out of track. In addition to state, we also add signal dir to record which direction it should turn before the car goes out of track. It can helps us decide on whether STAY_L and STAY_R the car should switch to.

▲ Figure 3.2: diagram of dir

At first, our state transition is simply based on sensors’ output signals. However, we discover that the car will randomly turn its direction before going back to track from the black area. It’s because of the scratching on black areas.

▲ Figure 3.3: scratching the black area

When the sensors passes the scratching, they will output 1’b1. The not-wanted outputs are like glitches. Therefore, after a long term of testing, we add debounce on sensors’ outputs and let the car keep its state on STAY_L and STAY_R until the sensors are all on track.

▲ Figure 3.4: sensor signals debounce and state design

3.3 Motor Part

In the motor part, we modified the motor module. We design different pwm combinations according to mode(state). For states of one sensor out, we set one of the wheel’s pwm to 10’d1023(100%) and the other wheel’s to 10’d700(68.4%). For states of two sensors out, we set one of the wheel’s pwm to 10’d1023(100%) and the other wheel’s to 10’d600(58.7%). For states of all sensors out, we set both of the wheels’ pwms to 10’d1023(100%) and invert one of the wheel’s direction. In this way, the car can turn back to track faster.

▲ Figure 3.5: motor_part

3.4 Top Part

In the top part, we combine the above three parts, add some signals(LED_L, LED_M, LED_R, stop_b) for debugging, and change left motor’s and right motor’s direction according to the states and stop signals. When stop and stop_b are positive, {left, right} is changed to 4’d0.

When the car is in state STAY_L and STAY_R, {left, right} is changed to 4’b0110 and 4’b1001.

Under other conditions, {left, right} is set to 4’b1010. The overall design is shown in Figure 3.6.

▲ Figure 3.6: top part

What have we learned from Lab 6?

Haha

Contributions

謝佳晉：

范升維：